- FILE PATH: <PATH_to_USER_FOLDER>/esp/esp-idf/components/freertos/config/include/freertos/FreeRTOSConfig.h

- Preemption Scheduling

  - High priority task is executed and it will switch between equal priority tasks available in the ready task.

  - configUSE_PREEMPTION is the macro.

- Time Slicing

  - Here also it will run high high-priority task but it will not switch between equal tasks that are in ready state.

  - configUSE_TIME_SLICING is the macro.

- The attached ZIP folder consists of the code to get the details of the preemption bit and time-slicing.

- It consists of 2 tasks to operate 2 different LEDs

- The below scenario is performed and below are the results that occurred.

- Disable Preemption.

  - Make “configUSE_PREEMPTION” to “0” in “FILE PATH”

  - Executes successfully

- Enable preemption and remove the vTaskDelay from higher priority tasks only (so that it does not sleep).

  - Make “configUSE_PREEMPTION” to “1” in “FILE PATH”

  - The controller is resetting continuously.

  - Not going to Task 2.

- Remove vTaskDelays from both tasks and replace them with blocking loop-based delays or timer-based delays (so that both tasks are always active).

  - The controller is resetting.

  - A stack overflow in task blink_task1 has been detected

- Disable time-slicing and make both tasks’ priorities equal.

  - Make “configUSE_TIME_SLICING” to “0” in “FILE PATH”

  - The controller is resetting continuously.

  - ***ERROR*** A stack overflow in task blink_task2 has been detected

- Enable time-slicing.

  - Make “configUSE_TIME_SLICING” to “1” in “FILE PATH”

  - The controller is resetting continuously.

  - **ERROR*** A stack overflow in task blink_task1 has been detected.