- Understanding the “freeRTOSConfig.h” file.

  - PATH of file: vi $(HOME)/<Project/ApplicationName>/esp/esp-idf/components/freertos/config/include/freertos/FreeRTOSConfig.h

  - FreeRTOS is customised using a configuration file called FreeRTOSConfig.h.

  - Every FreeRTOS must have this file in the preprocessor PATH.

    - heap size

      - #define configTOTAL_HEAP_SIZE	10240

      - It gives the total amount of RAM available in the FreeRTOS heap.

    - stack size

      - #define configMINIMAL_STACK_SIZE	128

      - The size of the stack used by the idle task.

      - The stack size is specified in words, not bytes.

      - If each item placed on the stack is 32 bits, then a stack size of 100 means 400 bytes (each 32-bit stack item consuming 4 bytes).

    - CPU clock

      - #define configCPU_CLOCK_HZ                      60000000

      - frequency in Hz

      - The internal clock that drives the peripheral used to generate the tick interrupt will be executing

      - The same clock drives CPU clock

      - For configuring timer peripherals correctly, a clock value must be given.

    - tick rate

      - #define configTICK_RATE_HZ                      250

      - The frequency of the RTOS tick interrupt.

  - Link: freeRTOSConfig.h

  - It has Memory-allocated definitions, Tick Rate, Hook function-related definitions, and Software Timer Related definitions.

- Understanding Critical sections for ARM cortex FreeRTOS ports

  - Available Priority Levels

    - Cortex-M hardware details

      - Priorities are set by manufacturer that uses the ARM Cortex-M core, So, all ARM cortex have the same number of unique interrupt priorities

      - Allows Max 265 (8 bits - 0 to 0xff) different priorities from which some of them are allowed to use.

    - Relevance when using the RTOS

      - Two groups

        - that will get masked by RTOS critical sections

        - that are never masked by RTOS critical sections and are therefore always enabled.

      - configMAX_SYSCALL_INTERRUPT_PRIORITY defines the boundary between the above 2 groups.

  - Preempt Priority and Subpriority

    - Cortex-M hardware details

      - The 8-bit priority register is divided into two parts

        - preempt priority

          - defines whether an interrupt can preempt an already executing interrupt.

        - sub priority

          - determines which interrupt will execute first when two interrupts of the same preempt priority occur at the same time

    - Relevance when using the RTOS

      - It is recommended to assign all the priority bits to be preempt priority bits, leaving no priority bits as subpriority bits

  - Inverse Relationship Between Numeric Priority Value and the Logical Priority Setting

    - Cortex-M hardware details

      - 2 interrupts have been given priority levels as 2 and 5

      - The High priority is given to 2 levels and Low priority is given to 5 level

    - Relevance when using the RTOS

      - configMAX_SYSCALL_INTERRUPT_PRIORITY is defined in the FreeRTOSConfig.h header file

      - ISR define in RTOS API >= configMAX_SYSCALL_INTERRUPT_PRIORITY setting

      - To ensure interrupt logical priority <= configMAX_SYSCALL_INTERRUPT_PRIORITY

  - Cortex-M Internal Priority Representation

    - Cortex-M hardware details

      - ARM Cortex-M stores interrupt priorities in MSB of 8 bit interrupt priority.

      - Below image shows the detail for the same.

    - Relevance when using the RTOS

      - set by the configMAX_SYSCALL_INTERRUPT_PRIORITY

  - Critical Sections

    - Cortex-M hardware details

      - ARM Cortex-M core's BASEPRI register.

      - This allows the RTOS kernel to only mask a subset of interrupts, and therefore provide a flexible interrupt nesting model.

      - BASEPRI is a bit mask

    - Relevance when using the RTOS

      - The RTOS kernel creates a critical section by writing the configMAX_SYSCALL_INTERRUPT_PRIORITY value into the ARM Cortex-M BASEPRI register. As priority 0 interrupts (the highest priority possible) cannot be masked using BASEPRI, configMAX_SYSCALL_INTERRUPT_PRIORITY must not be set to 0.

- Hook Function

  - Hook Function Link

  - Idle Hook Function

    - void vApplicationIdleHook( void );

    - The idle hook will only get called if configUSE_IDLE_HOOK is set to 1 within FreeRTOSConfig.h.

    - An idle task can be called the Idle Hook Function.

    - The idle task can run at the lowest priority. Idle tasks are run when no higher-priority task is available.

  - Tick Hook Function

    - void vApplicationTickHook( void );

    - It provides a convenient place to implement timer functionality.

    - The Tick Hook function is called only if ConfigUse_Tick_Hook is set to 1 in freeRtosconfig.h

  - Malloc Failed Hook Function

    - Implementing Malloc Failed Hook function will identify the problem caused by lack of heap memory.

    - Malloc Failed Hook is called only if ConfigUse_Malloc_Failed_Hook is set to 1 in FreeRTOSConfig.h.

    - heap_1.c, heap_2.c, heap_3.c, heap_4.c and heap_5.c

      - heap_1 - the very simplest, does not permit memory to be freed.

      - heap_2 - permits memory to be freed, but does not coalescence adjacent free blocks.

      - heap_3 - simply wraps the standard malloc() and free() for thread safety.

      - heap_4 - coalescences adjacent free blocks to avoid fragmentation. Includes absolute address placement option.

      - heap_5 - as per heap_4, with the ability to span the heap across multiple non-adjacent memory areas.

    - void vApplicationMallocFailedHook( void );

  - Stack Overflow Hook Function

    - Stack Overflow Protection

  - Daemon Task Startup Hook

    - It allows initialization code before use of RTOS functionality.

    - same as the Timer Service Task.

    - Daemon Task StartUp Hook is called only if ConfigUse_Daemon_Task_startUp_Hook is set to 1 in FreeRTOSConfig.h

    - void vApplicationDaemonTaskStartupHook( void );

- The attached code is an example of the below parameters

  - Heap Size

  - Stack Size