- ESP32 Board Details

  - Product: Development Board

  - Model: ESP32-WROOM-32D

  - CPU:  Xtensa dual-core 32-bit LX6 microprocessor

  - Antenna: On board Antenna

  - Frequency: 240 MHz

  - Voltage: 3.6V

  - Current: 500mA

  - Core: ESP32-D0WD

  - SPI flash: 32 Mbits, 3.3 V

  - Crystal: 40 MHz

  - Antenna: on-board PCB antenna

- DataSheet

- Model: ESP32-WROOM-32D

- CPU clock frequency: 80 MHz to 240 MHz

- Microprocessor: Tensilica Xtensa LX6 dual-core

- Voltage (V): 3.6

- Current(mA): 500

- ROM: 448 KB ROM

- SRAM: 520 KB SRAM

- On Board Ports: Wi-Fi + Bluetooth + Bluetooth LE MCU

- Operating Temperature (°C):	-40 to 85 °C

- Shipping Weight: 0.01 kg

- Shipping Dimensions: 8 × 5 × 2 cm

- Use the below link to set up the ESP32 environment on the Ubuntu System.

  - ESP environment setup

- Use the below details for the setup

  - Board: ESP32

  - Branch: Master

- After Setting up the environment create an example using the below command

  - Go to the below path

    - $(PROJECT_FOLDER_PATH)/esp

  - Execute the below command to create a custom application

    - idf.py create-project <Project/Application Name>

    - A Folder will be created with “Project/Application Name”

    - Goto that folder and use the below command to edit the source code

      - cd <Project/Application Name>

      - vi main/<Project/Application Name>

      - Edit the code, then save and exit the code.

  - Setup Environment

    - . $(PROJECT_FOLDER_PATH)/esp/esp-idf/export.sh

    - Example: . $HOME/ESP32_Project/esp/esp-idf/export.sh

  - To Open Menuconfig use the below command

    - idf.py menuconfig

  - Execute the below command to compile, run, and monitor the application

    - Compile

      - idf.py build

    - Dump the code

      - sudo chmod a+rw /dev/ttyUSB0

      - idf.py flash

    - Monitor/Debug

      - idf.py monitor

    - Exit for IDF monitor

      - Ctrl+]

- Understood the SDK structure based on the ESP environment setup

- Below is the link that gives the details on how to access Pheriperials.

  - Peripherals Usage

- ZIP folder “esp32_led_control” contains the example for LED operation.