STM32F407 + SEN0528 (ATH20) -> RTOS I2C Driver

- Introduction

  - Statement - Develop a POC to write a driver code in RTOS to display Temperature and Humidity over console.

  - Connect ATH20 Sensor to STM32F407 Board and read temperature and humidity data.

  - Display the data on console

- Devices

  - Reference Board: STM32

  - DEV Kit: STM32F407-DISC2

  - I2C Sensor: DFROBOT - SEN0528 (Based on ATH20)

  - OS: RTOS

- Connection

| Sr. No. | STM32 Pin | ATH20 Pins | I2C Details |
| --- | --- | --- | --- |
| 1 | PB6 | C | Clock |
| 2 | PB9 | D | Data |
| 3 | Gnd | - | Supply Gnd |
| 4 | 3V | + | Supply Vcc |

- Code details

  - Called function snippet is shown below

void StartDefaultTask(void *argument) { for(;;) { HAL_Delay(40); //40 ms delay at boot uint8_t buffer[7] = {0}, status = 0xFF; I2C_ReadData(SLAVE_ADDRESS, 1, buffer, &status); //Check Calibration bit of Status if (status == 0) printf("Read OK: Calibration State %02X n", buffer[0]); else { buffer[0] = 0xBE; buffer[1] = 0x08; buffer[1] = 0x00; I2C_WriteData(SLAVE_ADDRESS, 3, buffer, &status); //Initialize Cal //Send 0xBE command HAL_Delay(10); } while(1) { buffer[0] = 0xAC; buffer[1] = 0x33; buffer[1] = 0x00; I2C_WriteData(SLAVE_ADDRESS, 3, buffer, &status); //Start the measurement command HAL_Delay(100); //80ms Delay if (status == 0) printf("Write OK\n"); memset(&buffer, 0x00, sizeof(buffer)); I2C_ReadData(SLAVE_ADDRESS, 7, buffer, &status); if (status == 0) printf("Read OK: %02X %02X %02X %02X %02X %02X %02X\n", buffer[0], buffer[1], buffer[2], buffer[3], buffer[4], buffer[5], buffer[6]); uint32_t RawTemp, RawHum; RawHum = ((buffer[1] << 12) ¦ (buffer[2] << 4) ¦ ((buffer[3] & 0xF0) >> 4)); RawTemp = ((buffer[3] & 0x0F) << 16) ¦ (buffer[4] << 8) ¦ (buffer[5]); printf(" HUm %f, Temp %f \r\n", Convert_Humidity(RawHum), Convert_Temperature(RawTemp)); HAL_Delay(2000); } } }

- Reference Driver files

  - I2C_Driver.c

  - I2C_Driver.h

- Status

  - Successfull read data from I2C sensor and display it on console.

- I2C Write Data Waveform

  - Configure I2C in Write mode by sending below command

    - (0x38 << 1)

  - Write Measurement Command as given below

    - 0xAC, 0x33, 0x00

- I2C Read Data Waveform

  - Configure I2C in Write mode by sending below command

    - (0x38 << 1) | 1

  - Read 6 bytes to get Temperature and Humidity data

  - Convert the bytes into data

- Reference Links

  - STM32F407 Reference Manual

  - STM32F407 Data Sheet

  - ATH20 Data Sheet